INSTALLATION REQUIREMENTS

- When replacing an existing machine, the services are normally in place and adequate.  However, it is advisable to check that the water supply and electrical services meet the current regulations.

- For sites not replacing an existing machine, the following services are required:-

1.	Properly rated electrical supply (within 1 metre).

Note - The machines are not supplied with a plug.

This enables the installer to ‘connect’ the machine to the supply. It does not negate the need for ‘appliance testing’ needs. (Hubbard Contractors will supply a plug if required)

2.	A ¾ ” washing machine type tap on the mains water supply (within 1 metre) this supplying ‘potable’ drinking water. Note – A tank fed supply to an ice machine is illegal

3.	A 1½” trapped and vented drain, no higher than the base of the machine (within 1 metre) *.

Note – WRAS now suggest that a suitable ‘backflow prevention’ device is installed to reduce the risk of contamination.

*EC/EF self-contained machines have an in-built waste pump to offer a solution when the drain is too high or too far away, alternatively an external condensate pump can be supplied at additional cost, ensure adequate space and additional mains electric socket is available.

POSITIONING OF EQUIPMENT

Minimum ambient	temp 10oC

Maximum ambient	temp 40oC

Production rates are based on 10oC ambient and 10oC water.

As ambient and water temperatures increase, ice production rates decrease.

Ensure sufficient space exists for airflow and services (150mm on all sides of machine is recommended).

ACCESS

Ensure the selected equipment will pass through doorways and can be manoeuvred around corners and stairways.

When turning equipment through 90o, the critical dimension is the diagonal.

CUSTOMER CONFIRMATION

I confirm that the responsibility for the above conditions being correct and available at the time of installation is ours. I also confirm that I am authorised to sign this document.

Signed …………………………………………………………………

Position ………………………………………………………………..

Company Name ………………………………………………………

Note – It is the customers responsibility to ensure the above conditions are met and ready on the arrival date of our refrigeration engineers. Should any of the above conditions not be correct an aborted installation charge will be levied payable by the customer.